# HN9600/9800

# Fallback Updater Release Notes

H55489

Build 8.3.1.21

Revision A.02

December 21, 2017

Hughes Network Systems, LLC
11717 Exploration Lane
Germantown, Maryland  20876
(301) 428-5500

All rights reserved.  This publication and its contents are proprietary to Hughes.  No part of this publication may be reproduced in any form or by any means without the written permission of Hughes Network Systems, LLC., 11717 Exploration Lane, Germantown, Maryland 20876.

Copyright © 2017 Hughes Network Systems, LLC

REVISION HISTORY

| Document Revision | Release # | Date of Issue | Scope |
| --- | --- | --- | --- |
| A.01 | 8.2.0.50 | October 25, 2015 | Create Original |
| A.02 | 8.3.1.21 | December 21, 2017 | Giotto ODU support |

Table of Contents

1.1	Release Contents	4

1.2	Included files	4

1.3	Required equipment	4

1.4	Updater Instructions	5

Release Details

## Release Contents

This package consists of release notes and installation procedures for the HughesNet Fallback Updater tool for the following remotes: HN9600/9800.  The intent of this fallback updater is to convert the remote platforms listed above to fallback image 8.3.1.21_PID.

What’s New in this Release since last ship authorized version.

Updates fallback.bin to release Dec 18 2017, 11:09:39 (8.3.1.21_PID)

SPR Fixes in this build since previous updater:

- 50154	Giotto ODU support

- 49936	VSAT General - Extended Ku Band Tx Code 25 Recovery

SBC.CFG - Version YAH21Dec2017

## Included files

If you have successfully unzipped the Fallback Updater zip file, the following files will be placed in the directory you have chosen:

- Hughes_Updater.exe - This is the PC executable that transfers the fallback.bin, sbc.cfg, and zipdb.gz to the remote’s flash file system.

- Hughes_Updater.exe.manifest – Required to run Hughes_Updater.exe

- fallback.bin – the remote executable that will be installed by the updater

- ReadMe_HN9600FBU_8.3.1.21.doc – this file

- sbc.cfg – Satellite Based Commissioning configuration file (Version= YAH21Dec2017)

- zipdb.gz – the zipcode database (USA compatibility)

#### nostart.dat - A control file loaded to the remote during fallback updater execution to force the remote to run a minimal set of tasks while the updating is in progress.  The file is ftp’d to the unit being updated and should be automatically removed after execution.  If the script is unexpectedly interrupted this file may be left on the unit. This causes the unit to appear unresponsive (brick), when in reality an ftp can be used to remove this file and restore the unit to normal operation (see “Troubleshooting” in Updater Instructions section).

- dummy – a small file used to perform initial testing of the ftp process between the PC and the remote.

## Required equipment

You can set up any number of stations to do this repair.  Each station must have:

•	Windows PC.  For this operation Windows 7 or newer is recommended.

•	Ethernet hub

•	Ethernet cables.  PC<-->hub, target<-->hub.

•	Window PC with Microsoft Visual C++ 2008 Redistributable Package (x86) – vcredistx86 installed

## Updater Instructions

This program runs on a Windows PC and can be used to update:

- HN9600

- HN9800

PROCEDURE TO SET UP ONE REPAIR STATION

STEP 1.  Unzip the HN9600FBU_8.3.1.21.zip updater.

STEP 2.  Configure your PC LAN port.  From the windows START button in the lower left hand corner of your computer screen select Settings->Control Panel->Network.  You should see screens similar to those directly below.  Select the Protocols tab.  For the TCP/IP Protocol, select Properties.  Select “Specify an IP address.”  Enter the following:

IP Address 192.168.0.2

Subnet Mask 255.255.255.0

Default Gateway 192.168.0.1

STEP 3.  Disable any firewalls that would block FTP or Telnet. The figure below shows an example of Windows XP disabled firewall (reached via Network Connections->Local Area Connection Properties (Advanced Tab)->Windows Firewall “Settings” button):

PROCEDURE TO UPDATE ONE TARGET

Repeat this procedure for each target to be updated:

- Connect the PC and remote via the LAN1 connector.

- Open the Windows Explorer and navigate to the default directory where the files were unzipped.

- Double-click on HUGHES_Updater.

Results

The following messages will be generated if the fallback update operation is successful.

STEP 1

User Input: Click on the Accept button to acknowledge the restricted use condition.

STEP 2

User Input: Click on the OK button to begin the update process.

STEP 3

User Input: None

STEP 4

.

.

.

User Input: None

After the above operation is completed the user interface automatically exits. After the unit boots up, verify that the banner date of the software is correct (ignore the “TELNET_SendCommand:  read failed” message in the err.log file).